# Team Tetrahedron

# Milestone #4 -

# Data Analysis

Team members:

Lauro Fialho Müller

Chandan Radhakrishna

Raghava Vinaykanth Mushunuri

Kavya Vajja

Arnab Das

Anjan Chatterjee

26.05.2020

# Table of contents

Overview	2

Input variables measured	2

Output variables measured	2

Erich-Weinert-Str	3

Experimental observations and final conclusions	4

Am Fuchsberg	5

Experimental observations and final conclusions	6

Leipziger Str. South	7

Experimental observations and final conclusions	8

Leipziger Str. North	9

Experimental observations and final conclusions	10

Traffic lights	11

Bicycles, pedestrians and trams	12

For Trams	13

Car Moving Direction with Count & Probability	13

Additional measurements	16

Inputs:	16

Outputs:	16

Difficulties encountered while obtaining the data	17

Dealing with trade-off between good estimation of distribution function and variance	17

Limitations on the accuracy or validity of the data	20

Cost overview	21

Future work	22

# Overview

The data analysis in conducted this milestone. The main motivation here is to identify the theoretical distributions that best fit the data so that we can use them when building our simulation model. Other elements such as pedestrian and traffic light modelling are also considered.

# Input variables measured

- Inter arrival times are measured as inputs for estimating the distribution functions.

- Total bicycles and pedestrians entering  the system  per hour.

# Output variables measured

- Frequency of observed values for  k number of bins

- Expected frequency for each distribution type for k number of bins

- Inverse values of cumulative distribution function is measured for each distribution type considered.

- Underlying distribution functions for  the vehicles entering each lane of the intersection point.

- Statistics of each distribution is obtained

- Chi square values for each distribution type taken at every possible k value

(that is in the range of k = n  to n/5, n is total number of samples)

- Probabilities for the vehicle to take a turn is measured.

# Erich-Weinert-Str

Histogram:

QQ plot:

## Experimental observations and final conclusions

Observations:

- Distribution type: Lognormal

- Statistics:

  - Mean = 1.161

  - Standard deviation = 0.533

  - For  k = 31, chi-square calculated less than critical

From the above comparisons we can conclude the test failed to reject the null hypothesis and the observed values and Expected values have correlation and the expected values are not occurring just by a mere coincidence.

The expected data is following the observed data with 95% confidence level.

Conclusion :

Lognormal with mean = 1.161, standard deviation = 0.533

# Am Fuchsberg

Histogram:

QQ plot:

## Experimental observations and final conclusions

Observations:

- Distribution type: Lognormal

- Statistics:

  - Mean = 1.267

  - Standard deviation = 0.593

  - For  k = 27, chi-square calculated less than critical

From the above comparisons we can conclude the test failed to reject the null hypothesis and the observed values and Expected values have correlation and the expected values are not occurring just by a mere coincidence.

The expected data is following the observed data with 95% confidence level.

Conclusion :

Lognormal with mean =1.267, standard deviation = 0.593

# Leipziger Str. South

Histogram:

QQ plot:

## Experimental observations and final conclusions

Observations:

- Distribution type: Lognormal

- Statistics:

  - Mean = 0.662

  - Standard deviation = 0.871

  - For  k = 34, chi-square calculated less than critical

From the above comparisons we can conclude the test failed to reject the null hypothesis and the observed values and Expected values have correlation and the expected values are not occurring just by a mere coincidence.

The expected data is following the observed data with 95% confidence level.

Conclusion:

Lognormal with mean = 0.662, standard deviation = 0.871

# Leipziger Str. North

Histogram:

QQ plot:

## Experimental observations and final conclusions

Observations:

- Distribution type: Lognormal

- Statistics:

- Mean = 1.511

- Standard deviation = 1.112

- For  k = 25, chi-square calculated less than critical

From the above comparisons we can conclude the test failed to reject the null hypothesis and the observed values and Expected values have correlation and the expected values are not occurring just by a mere coincidence.

The expected data is following the observed data with 95% confidence level.

Conclusion:

Lognormal with mean = 1.511, standard deviation = 1.112

# Traffic lights

# Bicycles, pedestrians and trams

For Erich-Weinert-Str to Am Fuchsberg:

- Distribution type: Uniform-discrete [9  sec,34 sec]

- Minimum value = 386/hour (9 sec)

- Maximum value = 106/hour (34 sec)

For Am Fuchsberg to Erich-Weinert-Str:

- Distribution type: Uniform-discrete [10sec, 20sec]

- Minimum value = 393/hour (3600/393 = 10/sec)

- Maximum value = 186/hour (3600/186 = 20/sec)

For Leipziger-str south to Leipziger-str North:

- Distribution type: Uniform-discrete [10sec, 27sec]

- Minimum value = 363/hour (10sec )

- Maximum value = 137/hour (27sec)

For Leipziger-str North to Leipziger-str south:

- Distribution type: Uniform-discrete [12 sec, 28 sec]

- Minimum value = 290/hour (12 sec)

- Maximum value = 129/hour (28 sec)

## For Trams

- Distribution: Truncated normal

- Min value = 8 minutes, Max value = 12 minutes,

- Standard deviation = 2 minute, mean = 10minutes

# Car Moving Direction with Count & Probability

Data Provided by Landeshauptstadt MAGDEBURG

Year : 2015

Am Fuchsberg 		to		Leipziger strasse South		300

Leipziger strasse North		1250

Straight to Erich-Weinert-Str	3950

Total					5510

P1 = 300/5500 = 0.05

P2 = 1250/5510 = 0.23

P3 = 3950/5510 = 0.72

Erich-Weinert-Str 		to 		Leipziger strasse North		200

Leipziger strasse South		1400

Straight to Am Fuchsberg		3550

Total					5150

P4 = 200/5150 = 0.04

P5 = 1400/5150 = 0.27

P6 = 3550/5150 = 0.69

Leipziger Str. South 	to 		Am Fuchsberg			4100

To Erich-Weinert-Str		1250

Straight to Leipziger Str. North	2950

Total					8300

P7 = 4100/8300 = 0.49

P8 = 1250/8300 = 0.15

P9 = 2950/8300 = 0.36

Leipziger Str. North 	to 		Am Fuchsberg			750

To Erich-Weinert-Str 		600

Straight to Leipziger Str. South	2300

Total					3650

P10 = 750/3650 = 0.21

P11 = 600/3650 = 0.16

P12 = 2300/3650 = 0.63

# Additional measurements

## Inputs:

- Tram times

- Pedestrian traffic lights

## Outputs:

- Sum of squared errors to check how well the expected value fits with the actual value curve.

# Difficulties encountered while obtaining the data

We were unable to collect the data.

While analyzing the data we have encountered the problem of outliers due to which proper estimation of the distribution function is not done as the distribution function tries to fit the entire data which really does not have any impact on the system. The expected values are not in any way related to the actual data points thus leading to a false estimate of distribution function.

# Dealing with trade-off between good estimation of distribution function and variance

Data censoring is done in order to reduce the variance and the mean is shifted to a point where the amount of data is high so that we can estimate a good fit for the data.

Following are the observations:

Leipziger str south:

Before removing:

- Variance: 145.705501

- Expon SSE: 22771.40234860699, rejected null hypothesis with 95% C.I

- Log norm SSE: 10547.130880170222, rejected null hypothesis with 95% C.I

- Norm SSE: 43318.018614922636, rejected null hypothesis with 95% C.I

## Data < 15

exp: close fit in q-q plot but Rejected null hypothesis with 95% confidence interval as chi value is not less than tabulated (close to theoretical ), sse = 96.99359997385118

Lognormal: Failed to  reject null hypothesis with 95% confidence interval at k = 34 chi =40.55014279825781 < 44.985 (df = 31)  (not only 34but for most of them), sse= 294.0773265479426

Norm: Rejected null hypothesis with 95% confidence interval, chi value is not less than tabulated, sse =959.3228009715183

Variance: 7.719009

## Data < 25

exp: Rejected null hypothesis with 95% confidence interval but close fit in qq plot, sse = 908.1043538692305

Lognormal: failed to reject null hypothesis at k = 80,  chi = 96.8853491051358<   98.484  (df = 77)and for few other values after 81, close fit in qq plot, sse = 1139.8242136414551

Norm: No proper fit and Rejected null hypothesis with 95% confidence interval, sse = 3620.8693617585723

Variance: 19.972214

## Data < 30

exp: close fit but Rejected null hypothesis with 95% confidence interval , chi value is not less than tabulated for any k , sse = 1506.8272257584504

Lognormal: failed to reject null hypothesis with 95% confidence level  at k=83 , chi = 97.01192955789713< 101.88(df = 80), sse =1458.0231606620443

Norm: Rejected null hypothesis with 95% confidence interval, chi value is not less than tabulated for any k , sse = 5141.926255269538

Variance: 26.079071

## Data < 40

exp: Rejected null hypothesis with 95% confidence interval, chi value is not less than tabulated for any k , sse = 4586.7407320703205

Lognormal: failed to reject null hypothesis at(k=91) = chi = 108.835 < 110.90(df = 88), sse = 3141.7580461553866

Norm: Rejected null hypothesis with 95% confidence interval,  chi value is not less than tabulated for any k , sse = 11457.69245570545

Variance:  48.390222

## CONCLUSION:

lognormal Data<15 would be a best choice as it failed to reject null hypothesis and q-q plot fits well with slope =1 for considerable range.

As the variance decreased to a greater extent the mean is shifted and does not consider the data which has very less impact on the system.

# Limitations on the accuracy or validity of the data

- The data is cleaned for the sole purpose of getting an optimum distribution function neglecting the data which does not really impact the system.

- The data is biased towards smaller interarrival times.

- Patterns in the data are likely to change over time and timely analysis has to be done.

- Pedestrian data is not precise enough to do the analysis so we have to go with uniform distribution.

# Cost overview

The chart below presents a two-dimensional breakdown of costs, aggregated by both milestone and individual members.

Additionally, the chart below shows the cumulative cost of the project so far. The orange bars represent the planned milestone costs, and the blue line the planned cumulative cost. The green bars represent the actual milestone costs, and the red line the actual cumulative cost.

# Future work

Complete the simulation model and validate the model.